PRACTICAL ROBOTICS FIELD GUIDE

# LeRobot SO-101 + SmolVLA

From camera setup and demonstration recording to training and autonomous evaluation

Outcome  Follow this guide to create a clean pick-and-place dataset, train SmolVLA, locate the saved checkpoint, and safely run one autonomous evaluation episode.

| Item | Working value |
| --- | --- |
| Robot | SO-101 follower arm |
| Teleoperator | SO-101 leader arm |
| Camera | /dev/video0, OpenCV, MJPG, 640 x 480 at 30 FPS |
| Task | Pick the cube and place it inside the box |
| Policy | SmolVLA |
| Validated final checkpoint | /home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second/checkpoints/030000/pretrained_model |

Prepared from the complete implementation and troubleshooting session | 31 July 2026

# How to use this guide

This is a start-to-finish operating procedure for a specific, proven setup. Commands are written for Bash on Ubuntu. Run one command block at a time, read the output, and do not move forward when a check fails.

Important scope  This guide assumes LeRobot and the conda environment named lerobot are already installed. It focuses on the complete workflow after installation: hardware, camera, data, training, and evaluation.

## The workflow at a glance

Verify hardware ports and camera formats.

Preview and position the camera.

Record a small test dataset before collecting the final dataset.

Inspect episodes and metadata.

Resume recording without changing camera or codec settings.

Exclude any bad episode during training.

Train SmolVLA for 30,000 optimizer steps.

Locate the 30K checkpoint and run one safe evaluation episode.

## Beginner vocabulary

| Item | Working value |
| --- | --- |
| Follower | The robot arm that executes actions. |
| Leader | The arm you move by hand to teleoperate the follower during data collection. |
| Episode | One complete demonstration or rollout, such as one cube pick-and-place attempt. |
| Dataset repo ID | A logical name used by LeRobot, for example local/pick_place_second_test. |
| Dataset root | The real folder on disk that stores metadata, data files, and video. |
| Checkpoint | A saved policy model produced during training. |
| Policy | The learned model that converts observations into robot actions. |
| FPS | Frames per second captured by the camera and dataset. |
| MJPG / H.264 | MJPG is used for camera capture; H.264 is used for saved dataset video. |

# 1. Safety before commands

Keep the robot on a stable surface with enough clearance in every direction.

Remove people, cables, pencils, loose cubes, and other obstacles from the arm's swept volume.

Keep a hand near the physical power switch or emergency stop during every first test.

Begin with one 30-second evaluation episode, not a long multi-episode run.

Do not rely only on software keys when the robot is moving dangerously; remove power if necessary.

Danger  A trained policy can produce unexpected motion. Test slowly, with low-risk object placement and immediate access to hardware power.

# 2. Confirm the Linux devices

Connect the leader, follower, and camera. In the working setup, the follower used /dev/ttyACM1, the leader used /dev/ttyACM0, and the camera used /dev/video0.

ls -l /dev/ttyACM* /dev/video*

Do not assume  USB device numbers can change after unplugging or rebooting. Confirm the ports before recording or evaluation.

# 3. Inspect and preview the camera

## 3.1 List supported camera modes

v4l2-ctl --device=/dev/video0 --list-formats-ext

The tested camera supported MJPG at 640 x 480, 1280 x 720, 1920 x 1080, and higher resolutions at 30 FPS. The final practical choice was 640 x 480 at 30 FPS because it greatly reduced the slow encoding delay between episodes.

## 3.2 Preview the exact final resolution

ffplay \
  -f v4l2 \
  -input_format mjpeg \
  -video_size 640x480 \
  -framerate 30 \
  /dev/video0

ffplay is a simple live video player supplied with FFmpeg. The -video_size value is width x height. It requests a supported capture mode; it does not physically change the camera lens or add scene content.

Before recording  Close ffplay and guvcview. Only one application should own /dev/video0.

## 3.3 Optional camera controls

v4l2-ctl -d /dev/video0 \
  --set-fmt-video=width=640,height=480,pixelformat=MJPG

v4l2-ctl -d /dev/video0 --set-parm=30

v4l2-ctl -d /dev/video0 \
  --set-ctrl=auto_exposure=1,exposure_time_absolute=100,gain=5,sharpness=140,power_line_frequency=1

v4l2-ctl -d /dev/video0 \
  --get-ctrl=auto_exposure,exposure_time_absolute,gain,sharpness,power_line_frequency

On this camera, focus_automatic_continuous and focus_absolute were not available. An 'unknown control' error means software focus is unsupported; use the physical lens focus, camera distance, lighting, and mounting angle instead.

## 3.4 Frame-composition checklist

The full reachable tabletop area is visible.

The cube and destination box remain visible throughout the motion.

The follower arm is visible, but does not hide the object during grasping.

The operator's body and unrelated objects do not enter the frame.

Lighting is steady and the cube edges are sharp enough to distinguish.

The camera mount cannot move during data collection, training evaluation, or later tests.

# 4. Understand dataset naming

Every local dataset uses two identifiers. Keep them consistent, but understand that they serve different purposes.

| Item | Working value |
| --- | --- |
| --dataset.repo_id | Logical LeRobot name. Example: local/pick_place_second_test |
| --dataset.root | Physical directory. Example: /home/postgrad/fahim/datasets_2/pick_place_second_test |

Common mistake  If the root is wrong or missing meta/info.json, LeRobot may fall back to Hugging Face Hub and produce a misleading 401 Repository Not Found error for local/... . Fix the local path first.

# 5. Record demonstrations

## 5.1 Start with two test episodes

cd ~/lerobot

lerobot-record \
  --robot.type=so101_follower \
  --robot.port=/dev/ttyACM1 \
  --robot.id=my_awesome_follower_arm \
  --robot.cameras="{front: {type: opencv, index_or_path: 0, width: 640, height: 480, fps: 30, fourcc: MJPG}}" \
  --teleop.type=so101_leader \
  --teleop.port=/dev/ttyACM0 \
  --teleop.id=my_awesome_leader_arm \
  --display_data=true \
  --display_compressed_images=true \
  --dataset.repo_id=local/pick_place_second_test \
  --dataset.root="/home/postgrad/fahim/datasets_2/pick_place_second_test" \
  --dataset.single_task="pick the cube and place it inside the box" \
  --dataset.num_episodes=2 \
  --dataset.episode_time_s=30 \
  --dataset.reset_time_s=5 \
  --dataset.vcodec=h264 \
  --dataset.push_to_hub=false

Shell syntax  A backslash must be the final character on its line. Do not place a space after it. A missing backslash causes the next option to run as a separate command.

## 5.2 What the timing options really mean

| Item | Working value |
| --- | --- |
| episode_time_s | Maximum active demonstration duration. |
| reset_time_s | Time allocated to physically reset the cube and robot between episodes. |
| Encoding time | Additional time used to read temporary frames and encode video. It is not included in reset_time_s. |

In this installed LeRobot version, video encoding blocks the transition between episodes. At 1920 x 1080 it took minutes. At 640 x 480 with H.264 it became much faster, but there can still be a visible wait.

Do not interrupt encoding  After Right Arrow or Escape, wait while Map and libx264 messages finish. Repeated Ctrl+C can interrupt frame reading, cleanup, and metadata finalization, leaving a partial dataset or a core dump.

## 5.3 Recording keys

| Item | Working value |
| --- | --- |
| Right Arrow | Finish the current phase and proceed. It cannot bypass video encoding. |
| Left Arrow | Rerecord the current/latest episode before moving on. |
| Escape | Stop recording, then allow the current episode to encode and the devices to disconnect cleanly. |

You cannot use the recorder to jump backward from episode 15 and replace already-saved episode 14. Either use a dataset-editing tool supported by your version, append a replacement demonstration, or skip the bad episode during training.

# 6. Validate the dataset before collecting many episodes

## 6.1 Find the real dataset root

find "/home/postgrad/fahim/datasets_2" \
  -type f -path "*/meta/info.json" -print

## 6.2 Read dataset totals

python - <<'PY'
from pathlib import Path
import json

base = Path("/home/postgrad/fahim/datasets_2")
for path in base.rglob("meta/info.json"):
    info = json.loads(path.read_text())
    print(f"Dataset: {path.parent.parent}")
    print(f"Episodes: {info.get('total_episodes')}")
    print(f"Frames: {info.get('total_frames')}\n")
PY

## 6.3 Visualize an episode

lerobot-dataset-viz \
  --repo-id local/pick_place_second_test \
  --root "/home/postgrad/fahim/datasets_2/pick_place_second_test" \
  --mode local \
  --episode-index 0 \
  --display-compressed-images=true

Review image clarity, full object visibility, smooth robot state/action traces, task completion, and whether the final placement is actually inside the box. The Rerun message 'Reached memory limit ... dropping oldest data' affects only visualization history, not the saved dataset. Closing the viewer releases its memory.

# 7. Resume an existing dataset

Use exactly the same repo ID, root, camera name, resolution, FPS, codec, robot IDs, and task text. dataset.num_episodes is the number of additional episodes to record in that run.

cd ~/lerobot

lerobot-record \
  --robot.type=so101_follower \
  --robot.port=/dev/ttyACM1 \
  --robot.id=my_awesome_follower_arm \
  --robot.cameras="{front: {type: opencv, index_or_path: 0, width: 640, height: 480, fps: 30, fourcc: MJPG}}" \
  --teleop.type=so101_leader \
  --teleop.port=/dev/ttyACM0 \
  --teleop.id=my_awesome_leader_arm \
  --display_data=true \
  --display_compressed_images=true \
  --dataset.repo_id=local/pick_place_second_test \
  --dataset.root="/home/postgrad/fahim/datasets_2/pick_place_second_test" \
  --dataset.single_task="pick the cube and place it inside the box" \
  --dataset.num_episodes=2 \
  --dataset.episode_time_s=90 \
  --dataset.reset_time_s=10 \
  --dataset.vcodec=h264 \
  --dataset.push_to_hub=false \
  --resume=true

Compatibility rule  Do not switch resolution or video codec partway through the same dataset. Create a new dataset when capture settings change.

# 8. Prepare 71 episodes for training

The final dataset contained 71 zero-based episode indices: 0 through 70. Episode 14 was known to be bad, so the training command used every episode except 14, for a total of 70 episodes.

EPISODES="$(python -c 'import json; print(json.dumps([i for i in range(71) if i != 14]))')"

To train all 71 episodes, remove both the EPISODES assignment and the --dataset.episodes option. Removing only one of them is incomplete.

# 9. Train SmolVLA

## 9.1 Final training command

cd ~/lerobot

EPISODES="$(python -c 'import json; print(json.dumps([i for i in range(71) if i != 14]))')"

PYTORCH_CUDA_ALLOC_CONF=expandable_segments:True lerobot-train \
  --policy.path=lerobot/smolvla_base \
  --dataset.repo_id=local/pick_place_second_test \
  --dataset.root="/home/postgrad/fahim/datasets_2/pick_place_second_test" \
  --dataset.episodes="$EPISODES" \
  --rename_map='{"observation.images.front":"observation.images.camera1"}' \
  --policy.empty_cameras=0 \
  --policy.resize_imgs_with_padding='[256,256]' \
  --policy.use_amp=true \
  --policy.device=cuda \
  --batch_size=1 \
  --num_workers=0 \
  --steps=30000 \
  --policy.scheduler_decay_steps=30000 \
  --save_freq=5000 \
  --log_freq=10 \
  --output_dir="/home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second" \
  --job_name=smolvla_pick_place_second \
  --wandb.enable=false \
  --policy.push_to_hub=false

Camera-name bridge  Recording stored observation.images.front. Training renamed it to observation.images.camera1. Evaluation must therefore provide a camera named camera1.

## 9.2 Understand the training log

| Item | Working value |
| --- | --- |
| step:30K | The configured 30,000 optimizer updates were reached. |
| smpl:30K | Approximately 30,000 samples were used because batch_size was 1. |
| loss | Training objective. Normal small fluctuations are expected. |
| grdn | Gradient norm. Values around the observed 0.5-1.8 were stable. |
| lr | Learning rate. It decreased toward 2.5e-06 as scheduled. |
| ep | Roughly episodes' worth of samples, not the number of dataset episodes loaded. |
| epch:0.41 | About 41% of the dataset's total frame count was sampled by 30K steps. |

A successful finish was confirmed by these final messages:

Checkpoint policy after step 30000
End of training

Reality check  Low training loss does not guarantee reliable robot behavior. Evaluate multiple checkpoints and object positions under supervision.

# 10. Locate and verify checkpoints

find "/home/postgrad/fahim/outputs_second" \
  -name pretrained_model \
  -printf '%T@ %p\n' | \
  sort -nr

The verified latest checkpoint is:

/home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second/checkpoints/030000/pretrained_model

Other saved checkpoints were 005000, 010000, 015000, 020000, and 025000. Keep them until the robot has been evaluated; an earlier checkpoint can sometimes behave better than the final one.

# 11. Run one autonomous evaluation

Close camera preview applications, put the robot and cube in a safe starting state, and keep the emergency power control within reach.

cd ~/lerobot

EVAL_ID="eval_smolvla_$(date +%Y%m%d_%H%M%S)"

lerobot-record \
  --robot.type=so101_follower \
  --robot.port=/dev/ttyACM1 \
  --robot.id=my_awesome_follower_arm \
  --robot.cameras="{camera1: {type: opencv, index_or_path: 0, width: 640, height: 480, fps: 30, fourcc: MJPG}}" \
  --teleop.type=so101_leader \
  --teleop.port=/dev/ttyACM0 \
  --teleop.id=my_awesome_leader_arm \
  --policy.path="/home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second/checkpoints/030000/pretrained_model" \
  --policy.empty_cameras=0 \
  --policy.resize_imgs_with_padding='[256,256]' \
  --policy.use_amp=true \
  --policy.device=cuda \
  --display_data=true \
  --display_compressed_images=true \
  --dataset.repo_id="local/${EVAL_ID}" \
  --dataset.root="/home/postgrad/fahim/datasets_2/${EVAL_ID}" \
  --dataset.single_task="pick the cube and place it inside the box" \
  --dataset.num_episodes=1 \
  --dataset.episode_time_s=30 \
  --dataset.reset_time_s=5 \
  --dataset.vcodec=h264 \
  --dataset.push_to_hub=false

Required name  When a policy is provided, this LeRobot version requires the dataset name to begin with eval_. A name such as smolvla_eval_... fails. eval_smolvla_... works.

The leader is included because this installed version can otherwise remain stuck during reset when a policy is present but no teleoperator exists. During the active episode, the policy controls the follower; the leader supports the reset workflow.

# 12. Evaluate performance methodically

Run one episode with the cube near the center of the demonstrated workspace.

Check whether the gripper approaches, grasps, transports, and releases in the correct order.

Repeat with several cube positions represented in the training demonstrations.

Compare checkpoints 015000, 020000, 025000, and 030000 if final behavior is poor.

Count successes out of a fixed number of trials instead of relying on one lucky attempt.

Record failure categories: missed grasp, collision, premature release, box miss, or stalled motion.

# 13. Troubleshooting reference

| Symptom | Meaning and fix |
| --- | --- |
| Reset says 5 seconds but takes minutes | reset_time_s excludes frame conversion and video encoding. Use 640 x 480, H.264, and wait for encoding to finish. |
| Right Arrow seems ignored | The key may have ended the phase, but encoding still blocks. Do not keep pressing keys. |
| Escape takes time | Escape requests a clean stop; the current episode is still finalized and encoded. |
| Invalid vcodec libx264 | This version accepts h264, hevc, or libsvtav1. Use --dataset.vcodec=h264. |
| 401 Repository Not Found for local/... | The local root is wrong or meta/info.json is missing. Find the actual metadata file and correct --root. |
| unknown control focus_absolute | The camera exposes no software focus control. Adjust physical focus and scene geometry. |
| Reached memory limit in Rerun | The viewer is dropping old displayed data. The dataset remains intact. |
| No checkpoints/last | This version saved numbered directories. Search for pretrained_model and use checkpoints/030000/pretrained_model. |
| outputs2 directory missing | The real root was /home/postgrad/fahim/outputs_second, not outputs2. |
| Dataset name does not begin with eval_ | Set EVAL_ID="eval_smolvla_$(date +%Y%m%d_%H%M%S)". |
| Vulkan/EGL viewer warnings | If the viewer opens and data loads, these graphics-backend warnings are generally harmless. |

# 14. Memory and disk-space checks

free -h
nvidia-smi
df -h

When the training process ends and the shell prompt returns, Linux automatically releases training RAM and GPU memory. Desktop processes may continue to use a small amount of GPU memory. Disk space is not automatically reclaimed because datasets and checkpoints remain saved.

du -h --max-depth=2 \
  "/home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second/checkpoints" | \
  sort -h

Keep your work  Do not delete the dataset or the 30K checkpoint before successful evaluation. Keep intermediate checkpoints until comparison is complete.

# 15. Final operator checklist

LeRobot environment is active.

Follower is confirmed on /dev/ttyACM1 and leader on /dev/ttyACM0.

Camera is /dev/video0 and no preview application is using it.

Camera frame is stable, clear, and matches training geometry.

Capture is 640 x 480 at 30 FPS using MJPG.

Dataset video codec is h264.

Recording camera is front; training renames front to camera1; evaluation camera is camera1.

Bad episode 14 is excluded from training when required.

Training ends with Checkpoint policy after step 30000 and End of training.

Evaluation dataset ID begins with eval_.

Only one 30-second evaluation episode is attempted first.

Emergency power control is immediately accessible.

# Appendix Proven paths and names

| Item | Working value |
| --- | --- |
| LeRobot working directory | /home/postgrad/lerobot |
| Dataset root | /home/postgrad/fahim/datasets_2/pick_place_second_test |
| Dataset repo ID | local/pick_place_second_test |
| Training output | /home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second |
| Final policy | /home/postgrad/fahim/outputs_second/train/smolvla_pick_place_second/checkpoints/030000/pretrained_model |
| Follower port | /dev/ttyACM1 |
| Leader port | /dev/ttyACM0 |
| Camera device | /dev/video0 |

Final principle  Change one variable at a time. Validate two episodes before recording dozens, validate the dataset before training, and validate one rollout before attempting many.