PRACTICAL SETUP GUIDE

SO-101 with LeRobot

Install, configure, calibrate, teleoperate, record data,
train SmolVLA, and run an evaluation

Example port map used in this guide: Leader = /dev/ttyACM0; Follower = /dev/ttyACM1. Always verify after reconnecting USB devices.

Target platform

Ubuntu 22.04 x86_64 | Python 3.10 | CUDA 12+ | PyTorch 2.6+

Validated against the linked references on 30 July 2026

# 1. Read this first

This guide is written for a single SO-101 leader/follower pair and one OpenCV camera. Commands are ready to copy, but device ports, camera index, storage paths, and task names must match your computer.

Robot safety: Clamp both arms securely, clear at least 1 metre around them, keep hands out of the motion area, and power off before changing servo, USB, or power wiring. Stop immediately if the arm shakes, collides, disconnects, or makes abnormal noise.

| Item | Value / meaning |
| --- | --- |
| Leader arm | Human-operated arm. Example port: /dev/ttyACM0; calibration ID: my_awesome_leader_arm. |
| Follower arm | Powered arm that copies the leader. Example port: /dev/ttyACM1; calibration ID: my_awesome_follower_arm. |
| Calibration ID | Name used to locate saved calibration. Reuse the exact same ID in calibration, teleoperation, recording, and evaluation. |
| Ports | USB device names assigned by Linux. They can swap after unplugging, rebooting, or changing USB order. |
| Dataset root | /home/postgrad/fahim/datasets/pick_place_v2 in the examples. |
| Training output | /home/postgrad/fahim/outputs/train/smolvla_pick_place_safe in the examples. |

# 2. Install LeRobot on Ubuntu 22.04

## Step 1 - Install Miniforge (one time)

Miniforge provides Conda, which keeps LeRobot and its Python packages isolated from the system Python.

wget https://github.com/conda-forge/miniforge/releases/latest/download/Miniforge3-Linux-x86_64.sh chmod +x Miniforge3-Linux-x86_64.sh ./Miniforge3-Linux-x86_64.sh # After the installer finishes: source ~/.bashrc conda init --all

New terminals: The source and conda init commands are initial setup only. Later, open a terminal and run conda activate lerobot; no repeated conda init or source command is needed.

## Step 2 - Create the Python 3.10 environment

conda create -y -n lerobot python=3.10 conda activate lerobot

The first command creates the environment. The second makes its Python and packages active in the current terminal.

## Step 3 - Clone the Seeed LeRobot repository

git clone https://github.com/Seeed-Projects/lerobot.git ~/lerobot

This downloads the LeRobot source into ~/lerobot. If the directory already exists, update it with git pull instead of cloning again.

## Step 4 - Install system build tools and FFmpeg

sudo apt update sudo apt install -y build-essential conda install -y ffmpeg -c conda-forge

build-essential supplies GCC and related tools needed by packages such as evdev. FFmpeg handles dataset video encoding and also provides ffplay.

## Step 5 - Install LeRobot and Feetech motor support

cd ~/lerobot pip install -e ".[feetech]" # Required before SmolVLA training: pip install -e ".[smolvla]"

Editable installation makes the lerobot-* commands available while keeping the cloned source as the active package. The smolvla extra installs model-specific dependencies.

## Step 6 - Verify Python, Torch, and CUDA

python --version python -c "import torch; print('torch:', torch.__version__); print('cuda:', torch.version.cuda); print('available:', torch.cuda.is_available())" lerobot-train --help ¦ head

Expected: Python 3.10, Torch 2.6 or newer, CUDA 12 or newer, and available: True for GPU training. If False, install a CUDA-enabled PyTorch build that matches the GPU/driver before training.

# 3. Identify and permit the USB ports

## See connected serial devices

ls -l /dev/ttyACM* # Some controller boards may appear as: ls -l /dev/ttyUSB*

Connect one arm at a time if you are unsure which device belongs to which arm. You can also run lerobot-find-port and follow its unplug/replug prompt.

lerobot-find-port

## Grant read/write permission before operation

sudo chmod 666 /dev/ttyACM*

Permission scope: chmod 666 is temporary and may need to be repeated after reconnecting or rebooting. A persistent alternative is sudo usermod -aG dialout $USER, followed by logging out and back in.

# 4. Configure servo IDs (kit assembly only)

Use this stage for unconfigured motors. A pre-assembled, already configured arm can normally proceed to full-arm calibration. During motor setup, connect only the motor requested by the interactive tool and check the power/3-pin wiring before pressing Enter.

## Follower motor setup

lerobot-setup-motors --robot.type=so101_follower --robot.port=/dev/ttyACM1

## Leader motor setup

lerobot-setup-motors --teleop.type=so101_leader --teleop.port=/dev/ttyACM0

Important: The leader is a teleoperator, so its setup flag is --teleop.type. Port names shown during one-at-a-time setup may differ from the final two-arm port map.

# 5. Calibrate the follower and leader

Calibration maps each servo's raw position to the arm's usable motion range. Place every joint near the middle of its range before pressing Enter. Then move each joint through its full safe range when prompted.

Match the arms: For the best leader-to-follower alignment, start both arms in the same physical middle pose: corresponding leader and follower joints should be at the same angles.

## Calibrate the follower

lerobot-calibrate --robot.type=so101_follower --robot.port=/dev/ttyACM1 --robot.id=my_awesome_follower_arm

## Calibrate the leader

lerobot-calibrate --teleop.type=so101_leader --teleop.port=/dev/ttyACM0 --teleop.id=my_awesome_leader_arm

If an existing calibration is found, press Enter to reuse it or type c and press Enter to recalibrate.

## Inspect saved calibration files

cat ~/.cache/huggingface/lerobot/calibration/robots/so_follower/my_awesome_follower_arm.json cat ~/.cache/huggingface/lerobot/calibration/teleoperators/so_leader/my_awesome_leader_arm.json

## Delete a saved calibration only when you intend to replace it

# Preview matching files first: find ~/.cache/huggingface/lerobot/calibration -type f -name "*my_awesome_follower_arm*" -print find ~/.cache/huggingface/lerobot/calibration -type f -name "*my_awesome_leader_arm*" -print # Then delete the intended matches: find ~/.cache/huggingface/lerobot/calibration -type f -name "*my_awesome_follower_arm*" -delete find ~/.cache/huggingface/lerobot/calibration -type f -name "*my_awesome_leader_arm*" -delete

If the new calibration does not start, first confirm the port. If the port is correct, power off, reconnect USB and electrical connections, restore permissions, and try again.

# 6. Optional servo middle-position repair

Use this only if joints cannot be moved into a sensible centre pose or you receive a magnitude/zero-offset error. These scripts belong to Seeed_RoboController, not the LeRobot repository.

Install the helper tool once

cd ~ git clone https://github.com/Seeed-Projects/Seeed_RoboController.git cd Seeed_RoboController pip install -r requirements.txt

Run from ~/Seeed_RoboController

# 1) Disable torque so joints can be moved by hand: python -m src.tools.servo_disable # 2) Store the current physical position as the middle (2048): python -m src.tools.servo_middle_calibration # 3) Command the servos to 2048 and verify the arm returns there: python -m src.tools.servo_center_test

After repair: Return to the normal lerobot-calibrate procedure and recalibrate the complete arm. The helper's centre test may move the arm automatically; clear the workspace first.

# 7. Teleoperate from a new terminal

conda activate lerobot cd ~/lerobot sudo chmod 666 /dev/ttyACM* lerobot-teleoperate --robot.type=so101_follower --robot.port=/dev/ttyACM1 --robot.id=my_awesome_follower_arm --teleop.type=so101_leader --teleop.port=/dev/ttyACM0 --teleop.id=my_awesome_leader_arm

Moving the leader should now command the follower. The IDs must exactly match the saved calibration IDs. If the follower moves unexpectedly, stop the command, verify ports and calibration, and restart from a safe middle pose.

# 8. Check the camera and storage

## Find the camera index

python -m lerobot.find_cameras opencv # Or, where supported: lerobot-find-cameras opencv

Use the reported OpenCV index as index_or_path in recording and evaluation. The examples use camera 0.

## View the raw camera

ffplay -f v4l2 -video_size 640x480 -framerate 30 /dev/video0

This checks the Linux video stream without LeRobot. Press q to close ffplay.

## Run the project's custom camera viewer (if present)

python camera_rerun.py

camera_rerun.py is a local project script, not a standard LeRobot command. Run it only from the directory that contains the file.

## Check free disk space

df -h ~

Watch the Avail column. Robot datasets and encoded videos can consume substantial storage.

# 9. Record the local pick-and-place dataset

The command below records 80 episodes locally. The front camera is fixed at 640x480 and 30 FPS. Keep the camera, lighting, workspace, task wording, and object appearance consistent.

lerobot-record --robot.type=so101_follower --robot.port=/dev/ttyACM1 --robot.id=my_awesome_follower_arm --robot.cameras="{front: {type: opencv, index_or_path: 0, width: 640, height: 480, fps: 30}}" --teleop.type=so101_leader --teleop.port=/dev/ttyACM0 --teleop.id=my_awesome_leader_arm --display_data=true --dataset.repo_id=local/pick_place_v2 --dataset.root=/home/postgrad/fahim/datasets/pick_place_v2 --dataset.single_task="pick the cube and place it" --dataset.num_episodes=80 --dataset.push_to_hub=false

| Item | Value / meaning |
| --- | --- |
| --robot.cameras | Defines the camera name, backend, device index, resolution, and frame rate. |
| --display_data=true | Shows live robot/camera data during recording. |
| --dataset.repo_id | Logical dataset identifier stored in its metadata. |
| --dataset.root | Physical directory where the local dataset is saved. |
| --dataset.single_task | Language instruction attached to every episode. |
| --dataset.num_episodes | Number of episodes to record in this run. |
| --dataset.push_to_hub=false | Keeps the dataset local; nothing is uploaded. |

## Keyboard controls

Right Arrow: finish the current episode/reset early and continue to the next.

Left Arrow: cancel the current episode and record it again.

Esc: stop the whole recording session; LeRobot finalizes/encodes the recorded data. Esc is not a pause button.

## Resume after stopping

lerobot-record --robot.type=so101_follower --robot.port=/dev/ttyACM1 --robot.id=my_awesome_follower_arm --robot.cameras="{front: {type: opencv, index_or_path: 0, width: 640, height: 480, fps: 30}}" --teleop.type=so101_leader --teleop.port=/dev/ttyACM0 --teleop.id=my_awesome_leader_arm --display_data=true --dataset.repo_id=local/pick_place_v2 --dataset.root=/home/postgrad/fahim/datasets/pick_place_v2 --dataset.single_task="pick the cube and place it" --dataset.num_episodes=77 --dataset.push_to_hub=false --resume=true

Resume count: With --resume=true, --dataset.num_episodes means additional episodes to record, not the final dataset total. Use 77 only if 77 more episodes are actually needed.

# 10. Train SmolVLA

Training reads the local dataset, fine-tunes lerobot/smolvla_base, saves checkpoints every 500 steps, and keeps the model local. Start with batch size 1 to reduce GPU memory use.

cd ~/lerobot PYTORCH_CUDA_ALLOC_CONF=expandable_segments:True lerobot-train --policy.path=lerobot/smolvla_base --dataset.repo_id=local/pick_place_v2 --dataset.root=/home/postgrad/fahim/datasets/pick_place_v2 --rename_map='{"observation.images.front":"observation.images.camera1"}' --policy.empty_cameras=0 --policy.resize_imgs_with_padding='[256,256]' --policy.use_amp=true --policy.device=cuda --batch_size=1 --num_workers=0 --steps=10000 --save_freq=500 --log_freq=10 --output_dir=/home/postgrad/fahim/outputs/train/smolvla_pick_place_safe --job_name=smolvla_pick_place_safe --wandb.enable=false --policy.push_to_hub=false

| Item | Value / meaning |
| --- | --- |
| PYTORCH_CUDA_ALLOC_CONF | Uses expandable CUDA memory segments to reduce fragmentation. |
| --rename_map | Renames the dataset's front image feature to the camera1 feature expected by this training configuration. |
| --policy.resize_imgs_with_padding | Resizes camera frames to 256x256 without simple stretching. |
| --policy.use_amp=true | Uses automatic mixed precision to reduce GPU memory and often improve speed. |
| --batch_size=1 | Processes one sample batch at a time; safest for limited VRAM. |
| --num_workers=0 | Loads data in the main process; slower but easier to troubleshoot. |
| --steps / --save_freq | Runs 10,000 updates and writes a checkpoint every 500 steps. |
| --wandb.enable=false | Disables Weights & Biases logging. |

Version-sensitive options: LeRobot CLI options change between releases. Before a long run, check lerobot-train --help. Keep camera feature names consistent between dataset, rename_map, checkpoint configuration, and evaluation.

# 11. Run the trained policy safely

Evaluation runs the follower from the last saved policy checkpoint and records one evaluation episode in a timestamped folder. Remove the leader from the command because the policy supplies actions.

conda activate lerobot cd ~/lerobot sudo chmod 666 /dev/ttyACM* EVAL_ID="eval_smolvla_$(date +%Y%m%d_%H%M%S)" lerobot-record --robot.type=so101_follower --robot.port=/dev/ttyACM1 --robot.id=my_awesome_follower_arm --robot.cameras="{front: {type: opencv, index_or_path: 0, width: 640, height: 480, fps: 30}}" --policy.path=/home/postgrad/fahim/outputs/train/smolvla_pick_place_safe/checkpoints/last/pretrained_model --policy.input_features='{"observation.state":{"type":"STATE","shape":[6]},"observation.images.front":{"type":"VISUAL","shape":[3,480,640]}}' --policy.empty_cameras=0 --policy.resize_imgs_with_padding='[256,256]' --display_data=true --display_compressed_images=false --dataset.repo_id="local/${EVAL_ID}" --dataset.root="/home/postgrad/fahim/datasets/${EVAL_ID}" --dataset.single_task="pick the cube and place it" --dataset.num_episodes=1 --dataset.push_to_hub=false

First evaluation: Use a clear workspace, keep a hand near the emergency power switch, and be ready to stop the process. Start with one episode. If the camera feature name or state shape differs from the checkpoint configuration, correct it before commanding the arm.

# 12. Quick troubleshooting

| Item | Value / meaning |
| --- | --- |
| Cannot connect to port | Run ls -l /dev/ttyACM*, verify which arm is which, run sudo chmod 666 /dev/ttyACM*, and reconnect only while powered off. |
| Ports swapped | USB numbering is not permanent. Update every command or reconnect in the intended order, then verify again. |
| Calibration is ignored | At the prompt type c and Enter, or preview/delete only the intended calibration JSON, then rerun calibration with the same ID. |
| Arm cannot be centred by hand | Use servo_disable from Seeed_RoboController, position it safely, then perform middle calibration and centre test. |
| CUDA available is False | The environment has a CPU-only or mismatched Torch build. Install the CUDA-enabled PyTorch build for the installed driver/CUDA. |
| Out of GPU memory | Keep batch size 1 and AMP enabled; close GPU applications. expandable_segments helps fragmentation but cannot create more VRAM. |
| Camera fails or wrong view | Check /dev/video*, run the camera finder, test with ffplay, and update index_or_path. |
| Resume count is wrong | num_episodes is the number of additional episodes when --resume=true. |
| Training/evaluation feature error | Compare dataset camera keys and checkpoint config; front, camera1, rename_map, shapes, and resolution must agree. |
| Low disk space | Run df -h ~ before recording/training and free space outside the active dataset/output directories. |

# 13. Command meaning cheat sheet

| Item | Value / meaning |
| --- | --- |
| conda activate lerobot | Selects the isolated LeRobot Python environment in the current terminal. |
| cd ~/lerobot | Changes to the cloned LeRobot source directory. |
| lerobot-find-port | Interactively identifies a serial port by detecting disconnect/reconnect changes. |
| lerobot-setup-motors | Assigns/configures motor IDs and communication settings during hardware setup. |
| lerobot-calibrate | Measures joint centre/range data and saves it under the supplied arm ID. |
| lerobot-teleoperate | Reads leader positions and sends matching commands to the follower. |
| lerobot-record | Records robot state, actions, images, and metadata; with --policy.path it also runs policy inference. |
| lerobot-train | Trains or fine-tunes a policy from a LeRobot dataset. |
| ffplay | Displays a raw camera stream for a quick independent check. |
| df -h ~ | Shows disk usage in human-readable units for the home filesystem. |

# 14. References

Primary setup and operating reference: Seeed Studio - Getting Started with SO-ARM100/SO-ARM101 in LeRobot

Versioned SO-101 reference: Hugging Face LeRobot v0.6.0 - SO-101

Source repository used by this guide: Seeed-Projects/lerobot

Servo middle-position helper: Seeed-Projects/Seeed_RoboController

Reproducibility: These commands are tied to the installed repository and CLI version. Keep a record of git rev-parse HEAD and pip freeze once the setup works.

cd ~/lerobot git rev-parse HEAD pip freeze > lerobot-working-environment.txt